April 27, 2017

Jon Bridge

AEI

6 Wesley St

Chelmsford, MA 01824

RE: 	Gene Therapy Lab 4205 Renovations

Genzyme Framingham

4th Floor

49 New York Avenue

Framingham, MA

JOB 3323

Jon:

I am pleased to submit this quote for the plumbing work per the drawings provided (dated 3/25/17) and the following scope of work

Included in Quote

- Cut, cap and make safe demo as shown

- Connect to existing CO2, vac, low pressure nitrogen, commpressed air mains above ceiling in lab 4205

- Pipe new ½” CO2, low pressure nitrogen, compressed lines to supply equipment

- Pipe new ¾” vac line to supply new equipment

- Provide (1) CO2 drop to (GT-1) Shaker Incubator

- Provide (1) ¾” drop to (GT-2) 4’ Biosafety Cabinet

- Provide (1) ¾” vac, (1) ½” CO2, ½” Compressed air, (1) ½” LP Nitrogen, and (1) ½” Oxygen line to (GT-3) 6’ Biosafety Cabinet

- Provide (1) Vacuum drop to (GT-5) 4’ Biosafety Cabinet

- Provide (1) ½” CO2, (1) ½” Compressed Air, (1) LP Nitrogen, and (1) Oxygen drop to (GT-7) Future Bioreactor

- Furnish and install Watersaver color coded hoses to make final connections to biosafety cabinets

- Furnish and install Concoa 400 series regulators for CO2 supply to (1) GT-3 and (1) GT-7

- Install (2 total) panel mounted CO2 regulators for (1) GT-1 and (1) GT-6

- Reuse panel mounted CO2 regulators removed from 5th floor project

- Oxygen piping to be stainless steel tubing with Swagelok style compression fittings

- CO2, LPN2, CA, piping to be ACR tubing and oxygen cleaned fittings with brazed and purged joints

- Vacuum piping to be type L tubing with Pro Press fittings

- Lockout/Tagout (devices supplied by others)

Not Included in Quote

- Off hours work

- Protection

- Lab gas turrets

- Permit

- Furnish quick connect fittings for watersaver hoses

- Lab Gas regulators or manifolds

- Fire Department hot work detail